V2V HAZARD ALERT

Real-Time Vehicle-to-Vehicle Hazard Communication using ESP32 and LoRa

ABSTRACT

Road accidents remain one of the leading causes of fatalities worldwide, especially in regions where communication infrastructure is limited or unavailable. Conventional Vehicle-to-Vehicle (V2V) communication systems generally depend on cellular networks or expensive roadside infrastructure, making them less suitable for rural and low-connectivity environments.

The proposed project, V2V HAZARD ALERT, introduces an infrastructure-independent wireless hazard communication system using ESP32 microcontrollers and SX1278 LoRa modules. The system enables real-time communication between vehicles for transmitting emergency hazard alerts such as sudden braking, accidents, roadblocks, fog conditions, and emergency stops.

The project uses LoRa technology to establish long-range, low-power peer-to-peer communication between vehicles without internet connectivity. When a hazard is detected or manually triggered, the ESP32 generates a lightweight data packet and transmits it wirelessly through the LoRa module. Nearby vehicles receive the warning and notify drivers through OLED displays, LEDs, and buzzer alerts.

The system demonstrates reliable communication over large distances while maintaining low latency and low power consumption. The proposed solution is cost-effective, scalable, portable, and suitable for intelligent transportation systems, smart city applications, highways, campuses, and remote areas.

Keywords: Vehicle-to-Vehicle Communication, LoRa, ESP32, SX1278, Intelligent Transportation Systems, Embedded Systems, Wireless Communication, Hazard Detection, IoT.

1. INTRODUCTION

Road safety has become a major concern in modern transportation systems due to the increasing number of vehicles and traffic density. According to global transportation statistics, millions of accidents occur annually because drivers fail to receive timely hazard information. Delayed communication regarding road hazards such as accidents, sudden braking, poor visibility, or roadblocks significantly increases the probability of collisions.

Traditional Vehicle-to-Vehicle communication systems such as DSRC and Cellular V2X provide effective communication but require either roadside infrastructure or continuous cellular connectivity. These solutions are expensive, difficult to deploy, and less effective in remote regions.

To overcome these limitations, the proposed V2V HAZARD ALERT system uses LoRa-based wireless communication technology. LoRa provides long-range, low-power communication without requiring internet or GSM infrastructure. The system enables direct peer-to-peer communication between vehicles.

The project utilizes ESP32 microcontrollers integrated with SX1278 LoRa modules for wireless communication. The transmitter vehicle sends hazard alerts, while nearby receiver vehicles receive and display warnings instantly. The OLED display provides clear visual information, while buzzers and LEDs generate immediate audio and visual notifications.

The proposed system offers several advantages:

- Long-range communication

- Low power consumption

- Infrastructure independence

- Low deployment cost

- Real-time hazard notification

- Scalability for future expansion

This project demonstrates the practical implementation of embedded systems, IoT communication, and intelligent transportation technologies.

2. OBJECTIVES

The primary objective of the V2V HAZARD ALERT system is to develop a reliable and infrastructure-independent hazard communication system for vehicles using LoRa technology.

Specific Objectives

- To design a real-time Vehicle-to-Vehicle communication system.

- To establish long-range wireless communication using LoRa modules.

- To implement low-power embedded communication architecture.

- To provide immediate visual and audio alerts to drivers.

- To reduce accident probability through early hazard notifications.

- To develop a scalable and cost-effective transportation safety system.

- To enable communication without internet or GSM connectivity.

- To create a platform for future smart transportation applications.

3. LITERATURE REVIEW

Several wireless communication technologies have previously been used for Vehicle-to-Vehicle communication systems.

3.1 DSRC-Based Systems

Dedicated Short-Range Communication (DSRC) systems operate in the 5.9 GHz band and provide low-latency communication between vehicles. However, DSRC systems require dedicated roadside infrastructure and have limited performance in non-line-of-sight environments.

3.2 Cellular Vehicle-to-Everything (C-V2X)

C-V2X systems utilize cellular networks for communication. These systems offer high data rates and large coverage areas but depend heavily on network availability and involve operational costs.

3.3 IoT-Based Transportation Systems

Modern IoT transportation systems integrate sensors and cloud technologies for monitoring and analytics. However, many existing solutions are dependent on internet connectivity.

3.4 LoRa-Based Communication

LoRa technology has emerged as an efficient LPWAN solution for long-range wireless communication. LoRa offers:

- Long communication range

- Low power consumption

- Infrastructure independence

- Low operational cost

- High scalability

The proposed project uses LoRa technology to provide efficient hazard communication between vehicles.

4. SYSTEM ARCHITECTURE

The V2V HAZARD ALERT system consists of two main vehicle nodes:

- Transmitter Vehicle Node

- Receiver Vehicle Node

Each node contains:

- ESP32 Microcontroller

- SX1278 LoRa Module

- OLED Display

- Push Buttons

- LEDs

- Buzzer

- Power Supply

System Workflow

- Driver detects hazard.

- Hazard button is pressed.

- ESP32 processes input.

- LoRa module transmits alert packet.

- Receiver LoRa module receives packet.

- ESP32 decodes alert message.

- OLED display shows warning.

- Buzzer and LEDs activate.

System Architecture Diagram

Vehicle A

(Button/Sensor)

↓

ESP32

↓

SX1278 LoRa

)))))) Wireless Communication ))))))

SX1278 LoRa

↓

ESP32

↓

OLED + Buzzer + LEDs

Vehicle B

5. HARDWARE REQUIREMENTS

5.1 ESP32 Development Board

The ESP32 is a dual-core microcontroller with integrated Wi-Fi and Bluetooth capabilities. It is widely used in IoT and embedded systems applications.

Features

- Dual-core processor

- Low power consumption

- Multiple GPIO pins

- SPI and I2C support

- High processing capability

Role in Project

- Processes hazard inputs

- Controls LoRa communication

- Drives OLED display

- Controls LEDs and buzzer

5.2 SX1278 LoRa Module

The SX1278 is a long-range wireless communication module operating in sub-GHz frequency bands.

Features

- Long communication range

- Low power operation

- Peer-to-peer communication

- High receiver sensitivity

- Reliable data transmission

Role in Project

- Transmits hazard alerts

- Receives wireless packets

- Enables infrastructure-independent communication

5.3 OLED Display

The OLED display provides real-time hazard information to the driver.

Features

- Low power consumption

- High contrast display

- Compact design

- I2C communication support

Role in Project

- Displays warning messages

- Shows hazard status

- Provides driver information

5.4 Push Buttons

Push buttons are used for manual hazard triggering.

Hazard Types

- Accident alert

- Emergency braking

- Roadblock warning

- Fog alert

5.5 Buzzer and LEDs

The buzzer and LEDs provide immediate audiovisual alerts.

Functions

- Buzzer generates emergency sound

- LEDs indicate warning status

- Enhances driver awareness

6. SOFTWARE REQUIREMENTS

Software Tools

| Software | Purpose |
| --- | --- |
| Arduino IDE | Embedded programming |
| ESP32 Board Package | ESP32 support |
| LoRa Library | Wireless communication |
| Adafruit GFX | OLED graphics |
| SH110X Library | OLED display support |

Programming Language

The project is programmed using Embedded C/C++ in Arduino IDE.

Libraries Used

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SH110X.h>

7. CIRCUIT CONNECTIONS

LoRa Module Connections

| SX1278 Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SCK | GPIO 5 |
| MISO | GPIO 19 |
| MOSI | GPIO 27 |
| NSS | GPIO 18 |
| RST | GPIO 14 |
| DIO0 | GPIO 26 |

OLED Display Connections

| OLED Pin | ESP32 Pin |
| --- | --- |
| VCC | 3.3V |
| GND | GND |
| SDA | GPIO 21 |
| SCL | GPIO 22 |

Push Button Connections

| Button | GPIO |
| --- | --- |
| Hazard Button | GPIO 32 |
| Emergency Button | GPIO 33 |

Buzzer Connections

| Buzzer | GPIO |
| --- | --- |
| Positive | GPIO 25 |
| Negative | GND |

LED Connections

| LED | GPIO |
| --- | --- |
| Red LED | GPIO 2 |
| Green LED | GPIO 4 |

8. WORKING PRINCIPLE

The V2V HAZARD ALERT system operates using direct wireless communication between vehicles.

Step 1 – Hazard Detection

The driver presses a hazard button during emergency conditions such as:

- Sudden braking

- Accident

- Fog conditions

- Road obstruction

- Emergency stop

The ESP32 continuously monitors the input GPIO pins connected to the buttons.

Step 2 – Message Processing

When a hazard is detected:

- ESP32 creates a lightweight data packet.

- The packet includes:

  - Vehicle ID

  - Hazard type

  - Timestamp

  - CRC verification

This compact structure reduces communication delay.

Step 3 – Wireless Transmission

The SX1278 LoRa module transmits the packet wirelessly.

Communication Parameters

| Parameter | Value |
| --- | --- |
| Frequency | 433 MHz |
| Spreading Factor | 12 |
| Bandwidth | 125 kHz |
| Coding Rate | 4/5 |

LoRa technology enables long-range communication with low power consumption.

Step 4 – Alert Reception

The receiver vehicle continuously listens for LoRa packets.

When a valid packet is received:

- ESP32 decodes the message

- OLED displays warning text

- LEDs indicate emergency status

- Buzzer generates audio alerts

Step 5 – Driver Notification

The driver receives:

- Visual alerts

- Audio warnings

- Emergency indication

This provides additional reaction time and improves road safety.

9. ADVANTAGES

The proposed V2V HAZARD ALERT system offers several advantages compared to traditional communication systems.

Major Advantages

- Infrastructure-independent communication

- Long-range wireless communication

- Low power consumption

- Real-time hazard alerts

- Low deployment cost

- Compact embedded architecture

- Reliable communication

- Easy scalability

- Suitable for remote areas

- Fast response time

- Improved driver awareness

- Reduced accident probability

10. APPLICATIONS

The V2V HAZARD ALERT system can be used in multiple transportation and safety applications.

Applications

- Smart transportation systems

- Highway safety monitoring

- Campus transportation systems

- Emergency vehicle communication

- Rural transportation networks

- Intelligent traffic management

- Smart city applications

- Accident prevention systems

- Fleet management systems

- Autonomous vehicle support systems

11. RESULTS

The proposed system was tested in different environments to evaluate communication reliability.

Open Field Testing

- Reliable communication up to 600 meters

- Packets received up to 750 meters

- High packet delivery ratio

Campus LOS Testing

- Reliable communication up to 400–500 meters

- Minor signal attenuation due to obstacles

Campus NLOS Testing

- Reliable communication up to 300 meters

- Packet loss observed beyond range limits

Performance Metrics

- High communication reliability

- Low transmission latency

- Stable signal strength

- Efficient power utilization

The results demonstrate the feasibility of LoRa-based V2V communication for real-world transportation safety systems.

12. FUTURE SCOPE

The project can be enhanced further by integrating advanced technologies.

Future Enhancements

- GPS location tracking

- Automatic accident detection

- AI-based hazard prediction

- Mobile application integration

- Cloud connectivity

- Adaptive communication protocols

- Vehicle sensor integration

- Multi-vehicle networking

- Voice-based alerts

- Solar-powered vehicle nodes

Future improvements can transform the project into a complete intelligent transportation platform.

13. CONCLUSION

The V2V HAZARD ALERT system successfully demonstrates an infrastructure-independent Vehicle-to-Vehicle communication platform using ESP32 and LoRa technology. The proposed system provides efficient real-time hazard communication between vehicles without requiring cellular connectivity or roadside infrastructure.

The project achieves long-range communication with low power consumption and fast response time. The integration of OLED displays, buzzers, and LEDs improves driver awareness and safety.

Experimental testing confirms reliable communication performance under different environmental conditions including open-field, line-of-sight, and non-line-of-sight scenarios.

The system provides a low-cost, scalable, and practical solution for intelligent transportation systems and road safety applications.

The proposed project demonstrates the practical application of embedded systems, IoT communication, wireless networking, and intelligent transportation technologies.

REFERENCES

- World Health Organization, Global Status Report on Road Safety.

- IEEE Research Papers on Vehicle-to-Vehicle Communication.

- LoRa Communication Technology Documentation.

- ESP32 Technical Reference Manual.

- SX1278 LoRa Module Datasheet.

- Intelligent Transportation Systems Research Publications.

- Embedded Systems and IoT Research Journals.

AUTHORS

Kalle Uday Bhaskar

Department of Electronics and Communication Engineering

Kuppam Engineering College